This disk contains the distribution version of TheNet X-1J release 4.

1. Hardware Requirements

The software runs in a Z80 based TNC2 or similar clone such as the BSX2. It is installed as described in the bankswitch mods file, but essentially all it needs is a single piece of wire from pin 8 of the modem disconnect header to pin 1 of the EPROM.

There is also a version for the PK96 that needs no bankswitch mods but may need change for the DCDB select on pin 23

The EPROM needed is a 27512, rather than the 27256 of a normal TNC2. Pin 1 of the EPROM is bent out from the socket and connected as described above.

The TNC2 needs to be set to the higher clock speed of 4.9 MHz as a minimum. Whilst this version of TheNet is more efficient than others, it is not sensible to run at the slower speed.

The TNC needs at least 32K bytes of RAM. If a PK96 is used with extra RAM only 32 K will be used.

The TNC needs to have DCDB from the SIO/0 connected to pin 23 of the RS232 connector. See the section on RS232 later in this note for more information.

2. Installing over TheNet X previous versions

If you are replacing a previous ROM with TheNet in it, be sure to do a coldstart ( you may need to remove the battery link to force this ).

3. Files

The files on this disk are :

thenet1.x1j    Part 1 of the code

thenet2.x1j    part 2 of the code

readme.*       This file

configur.*     Installation guide

userguid.*     A user's guide to the node

overview.*     The sysop's manual

quickref.*     A handy quick reference guide

bankswit.*     Information on the hardware bankswitching

intro.*        The brief release note

smeter.*       Details on the Smeter hardware

devmeter.*     Details on the deviation meter

patch.exe      A windowing driven patcher

intel.exe      An Intel hex file dump utility

intel.c        The source of the above

motorola.exe   A Motorola S1 type file dumper

motorola.c     The source of the above

sethelp.exe    Utility to change the ROM help text

sethelp.c      The source of the above

printpwd.exe   Utility to print a password nicely

printpwd.c     The source of the above

smeter.csv     Sample data for smeter calibration

smeter.ljt     Circuit for an S meter in HP PCL format

ipxlink.doc    Ideas on node IP addressing

x1jconv.c      Source for x1jconv.exe

x1jconv.exe    Utility to help manage node configuration

x1jconv.txt    Documentation for x1jconv.exe

thenetx.hlp    Windows 3.1 Help file of main documents

The files denoted '*' are stored in two formats

.TXT	ASCII versions of the files

.DOC	Word for Windows 2.0 format

The files contain the same except for the DEVMETER and SMETER, as the ASCII version does not contain the circuit diagrams or graphs. The smeter circuit diagram is shown in laserjet printer format ( i.e. HPPCL ) in the file smeter.ljt. As this is a large file, it has been compressed into the zip file smeter.zip.

The file thenetx.hlp is a MS Windows 3.1 help file. Whilst running windows, open file manager, select the file and either run it or drag it to the desktop to create an Icon under windows that contains most of the main documentation.

4. Using it with a TCP/IP system

One of the reasons for the inclusion of the IP router was to help the development of IP networks. This is in two different ways :

	It allows a IP station that does not run 24 hours to run an IP router for the others in the area without leaving the PC running,

	It allows existing nodes to double as IP routers.

In scenario 1, where a station runs a TNC2 clone with a KISS ROM or similar, this software may be used instead of the KISS ROM. It should be configured to run KISS, Selective Copy on the RS232 port.

5. The other version

The version that includes the HIGH and LOW commands is not included here. It used a form of bankswitching that clashes with the deviation meter.

6. Problems

One of the main problems with release 2 was the loss of buffers over time. It was cured by a fault in the L2 receive handler when a reject frame was sent on connect establishment. In this case the receive buffer was not released. This has been cured.

One common problem. If your node appears to 'lose' nodes, look very carefully at the rates of node broadcasts and the setting of the algorithm control. The RS232 rate should be faster than the radio rate or it should be set to zero, and the alternative algorithm should only be enabled on the RS232 port if at all. Try switching it off to see if that helps.

The patcher has been altered. The code now comes as two parts and the patcher patches both at the same time. It needs & expects to be able to access both parts.

7.   RS232 connections

It is important that you understand the RS232 connections.

The RS232 port will operate in one of two basic modes, a dumb terminal mode and a protocol mode. Each of these modes has two basic types of operation. These are detailed further below.

The decision as to whether the node operates in dumb terminal or protocol mode depends on the setting of the DCDB input of the serial communications chip ( the SIO/0 ). In TNC2 clones, this is the signal on pin 23 of the RS232 connector, but in most TNCs you will need to either insert a link or a piece of wire, as detailed in your TNC manual. After connecting the links if needed, then leaving pin 23 open circuit will cause the TNC to operate in dumb terminal mode, and setting it to a negative voltage ( ie connecting it to pin 10 ) will cause it to operate in protocol mode.

If you use a BSX2 TNC, then you should connect pin 23 to pin 7 rather than pin 10 for protocol mode. You will also need to insert a link into JMP14 & make sure there is no jumper in JMP9.

If you use one of the other common TNC2 clones such as the PK80, MFJ1270, TNC-200 etc, then I understand ( but cannot confirm ) that you will need to connect a wire from JMP9 pin 1,2 or 3 - they are all joined on the PCB - to RS232 pin 23.

Incidentally, don't forget to increase the watchdog timer capacitor to give a 1 minute timeout ( C31 to 47uF in TNC2 clones ).

Finally, the protocol modes are CROSSLINK or KISS, selected by the firmware, and the dumb terminal modes are STANDARD or MODEM, again set by firmware. In the MODEM mode, hardware handshaking is used for connect request / disconnect. This is NOT the usual method of operation. Unless you are certain you understand, DO NOT use this mode !.

If you use the PK96 there are some problems with the RS232 port. See the text in the overview guide for details.

8. Introduction to nodes

If you have not used a TNC based node before, the following notes on the operation of the 'dumb terminal' port may be of use.

When pin 23 on the RS232 interface is high, and with a terminal or terminal emulator on the RS232 port, the node will not appear to respond to anything. If you send an 'escape' character, the node should respond with a '* '. If it does not, look at the TNC status LED. It should be dimly lit with a little bit of a flicker. If not, the node is not operating. If it is, you have an RS232 problem. If you receive unsolicited garbage whenever the TNC receives data, you have pin 23 tied low and you are receiving crosslink or kiss frames.

If it does respond with a star and a space, you can now type one of 3 characters - C, D or P.

P will display or set the current password.

D will disconnect you from the node.

C will connect you to the node.

Try the C command. You should get a 'connected' message. Hit '?' followed by return and you will get a list of all commands.

Escape commands only work on the RS232 port when in dumb terminal mode. Don't forget to disconnect from the node before you disconnect the terminal.

The main documentation details the new commands added to the original TheNet 1.01. It does not cover the original commands unless they have been changed. The commands of interest are :

Connect

Node

Route

User

CQ

Parms

Sysop

Info

Reset

These are described in user guide, with the exception of the Sysop aspects of the commands. These are as follows :

8.1 Node

The Node command may be used to make a manual entry in the Net/Rom node table. When this is done, it may also make an entry in the routes table if necessary. The syntax of the command is :

NODE Callsign + Ident Quality Count Port Neighbour

callsign is the callsign of the destination node

ident is the alias of the destination node

Quality is the node quality for the entry

Count is the obsolescence count to be given to the entry

Port is the level 2 port ( 0 for radio, 1 for RS232 )

Neighbour is the callsign of the neighbouring node to route through

So to make an entry that will never expire to ANODE:G8KBB-1 where the node is accessed directly on the radio port with a quality of 123, enter

NODE G8KBB-1 + ANODE 123 0 0 G8KBB-1

If the same station is not heard directly but is accessed through node GB7TXM-2 over the radio, and the entry will expire when its obsolescence drops to zero from an initial count of 8, enter :

NODE G8KBB-1 + ANODE 123 8 0 GB7TXM-2

An entry may be deleted by substituting '-' for '+'.

8.2 Route

The routing table tells the node the routes to neighbouring nodes. It can also be used to make a manual entry as follows :

ROUTE Port Callsign [ Digilist ... ] + pathquality

Port is the AX.25 port number, 0 for radio and 1 for RS232

Callsign is the neighbouring node's callsign

8.3 Parms

The PARMS command sets a number of basic node parameters. See the quick reference guide for a list of them.

8.4 Sysop

Sysop is used to attain Sysop status. The node will challenge with 5 numbers. These are used to look up the corresponding password characters in the Sysop password table. This password is returned to the node on the next line. The password is stored in the ROM and is copied into RAM following a coldstart. From a locally connected terminal the password may be displayed or changed. using the <escape>P command.

9. 'User channel' nodes

To create a 'user channel' node ( ie one that supports Net/Rom protocols on its RS232 port but not on its Radio port ), do the following :

	Set the radio port default quality to one less than minqual

	Bar node broadcasts on the radio port by setting MODE parameter number four to 2

	Ensure that no manual routes exist in the route table for port 0.

10. EADG Nodes

If you put a node up in the UK that can be heard from East Anglia, may I encourage you to put the QRA locator in the Beacon text to enable the G1TLH Beacon analysis program to report accurately on propagation conditions.

73's

Dave G8KBB @ GB7MXM.#36.GBR.EU

[44.131.16.31] g8kbb@g8kbb.ampr.org

7, Rowanhayes Close

Ipswich

IP2 9SX

England

Tel    0473 682266

+44 473 682266