Users

-----

For someone to be able to use the modem port, you MUST have given them a

password.  Do an EU and use W to set the password.  The user can change

it using the NP command over the modem port.

Modem Settings

--------------

The default settings are:  'T E0 V1 X1 Q0 S7=60 S0=0'

Do not change E0, Q0, V1, X1, or S0.  All other settings are fair game.

You can make any changes automatic by putting them in the

INITIALIZE_FILE (see PARMS.DOC)

Modem switches

--------------

You should set your modem to hangup and enter command mode when DTR goes

low.  On older modems, this is a hardware switch.  On newer modems, it

is a software switch (ATSxx=y type).

Some users have reported intermittent crashes during initialization with

certain modems.  A suggested fix is to

ECHO AT&C1&D2 > COMx

before bringing up the BBS.  The above line can be put in a BAT file.

Substitute a real com number for the "x" above.  This fix will only work

with COM1 and COM2.

User settings

-------------

Recommended user setting for their terminal programs are:

- Half duplex or Echo on -- Program must supply echo of what is being

typed.

Forwarding

----------

A typical forwarding script might be:

PATH AA6 B AA4RE

CT*70,8483865

R*call sign*

SAA4RE

R*password

SMYPASS

Note that this represents a change from previous versions.  The CONNECT

message is now taken care of by the code.  The system will also change

speed based on the connect message.

High speed modems

-----------------

I have test the BBS with a Telebit T2500 9600bps V.32, V.42bis modem

with good results.  Pointers are:

1) I locked the speed of the interface between the modem and the

computer.  I also set the "lock_modem_speed" variable to yes in

PARMS.BB.  This allows the modem to make whatever speed changes it wants

without have to keep resetting the speed in the BBS.

2) If you have an modem with error recovery or with compression, you

will probably need to set HARDWARE HANDSHAKING on in MBBIOS for the

modem port.  See MBBIOS documentation on how to do this.  Hint:  Its an

item on port definition panel.

3) When connecting to a Non-MNP modem, an MNP modem will try to

negotiate an MNP session.  This shows a garbage characters sent to the

distant (non-MNP) end.  You may have to change your forwarding scripts

to handle this.  Usually you can fix this by adding the following after

the connect line:

R*

S

Remember that all 9600 bps modems are not created equal.  There are

multiple implementations and none of them work with each other.  The two

most popular are HST and V.32.  V.32 is a CCITT standard for 9600 bps.

*** Warning *** Slow computers and some fast ones will get overruns at

9600 bps.  An overrun is when the line is delivering data to the

computer faster than the computer can handle.  This can be helped by

acquiring a serial port or modem card that has a 16550A chip in it

rather than the 8250 or 16450.  The 16550A is plug compatible with the

older chips and includes a 16-byte buffer.  You can purchase them for

about $10.

Cabling

-------

The following control lines are used on the modem cable:  RTS, CTS, DCD,

DTR, DSR.  Your RS-232 cable must have all of these.